Technical University  of Cluj-Napoca

Faculty of Electronics, Telecommunications and Information Technology

PROJECT

CAD TECHNIQUES

Student: Cîrnaț Ștefan-Darian

Year: II-ro

Specialization: Electronic Engineering , Telecommunications and  Information Technology

Group: 2126

Contained

1.	Project theme	3

1.1	Requirement	3

1.2	Design specifications	3

2.	Block diagram	4

3.	Wiring diagram  of the circuit	5

4.	Dimensioning of components (theoretical substantiation)	5

4.1	Sensor  polarization in current	5

4.2	Convert your domain	9

4.3	Setting threshold voltages	11

4.4	LED modeling	15

4.5	Central Relay	19

5.	Circuit simulation	20

5.1	Voltage on  temperature sensor	20

5.2	Voltage dome conversion	22

5.3	Simulation of thresholds	22

5.4	LED simulation	24

5.5	Monte-Carlo Analysis	24

5.6	Worst-Case Analysis	26

6.	Bibliography	28

# Project theme

The theme of the project is to make a  circuit for temperature control in a greenhouse.

## Requirement

Design  a temperature control system in a greenhouse. Knowing that the temperature   sensor used can  measure the temperature  linearly within the  range specified in  column E table,  the system shall be designed  so that  the   temperature in  the enclosure  remain  within the  range specified in column F. The temperature  sensor will polarize in the current. The linear variation of the  electrical resistance   of the temp sensor is specified in  column G and must be converted to a voltage variation  in the   range [0÷(Vcc- 2V)]. In the  premises,  the temperature is maintained within the  specified range by means of a  boiler controlled by a comparator and an  electromagnetic magnetic relay.  The central  heating-relay assembly will be  modeled using  a  resistor. The status of the boiler (on/off) is signalled by an LED, with the color specified in the table.

## Design specifications

| Measurable temperature range | Temperature in the premises [ ̊C] | Sensor resistance [kΩ] | VCC | LED color |
| --- | --- | --- | --- | --- |
| -30 .. +40 | -10...+20 | 13k - 23k | 13 | .MOV |

# Block diagram

# Wiring diagram  of the circuit

0

R11	VCC

Q4

Q2N2222

diodes1

LED

Power station

k

1

Rlim

301

0

Q4

Q2N2222

diodes1

LED

Power station

k

1

Rlim

301

0

Vo1

VCC

U5

LM741

+

3

-

2

V+

7

V-

4

OUT

6

1

BONE

1

2

BONE

5

Vref

R12

k

442

1

Meg

Vo1

VCC

U5

LM741

+

3

-

2

V+

7

V-

4

OUT

6

1

BONE

1

2

BONE

5

Vref

R12

k

442

1

Meg

Vo

Vo

0

PARAMETERS:	Vref	0

1k

# Dimensioning of components (theoretical substantiation)

## Sensor  polarization in current

The sensor is simulated using  a resistance with values in the  range [13k – 23k] [Ohm].  To do this  we will sweep the resistance in this range.

The polarization of the sensor is done with a current source taken from course 12 of  the CEF subject (link in bibliography). And the  sizing of the components  is based on the  calculation of the maximum current  that can pass through the   sensor without exceeding  the  range of (0, Vcc − 2), using  the following formula:

After calculations, a maximum current of 0.478mA is obtained.

The circuit for the voltage source is shown in  the image below:

We will size the current  source after this forum:

Following the calculations, we obtained a resistance value  RREE = 5.02kk [OOhmm]. In this calculation, we considered Zener diodes with  the following code: 1N5225 (rated voltage VV zz = 3.0V),  VV BBEE = 0.6  VV, and  I is the maximum  current calculated  above.

After sizing the current source, we will enter this current into a  current mirror (with a ratio of 1:1).

Thus,  the circuit that performs the  polarization of the resistive sensor  is shown below:

Fig. 1:  Power source  and current mirror

We chose a resistance (R2 – belongs to  a  standardized series) 0.2k Ohms  higher so as not to  reach the maximum current calculated above.

Following a  DC Sweep  simulation,   there   is   a very small variation  in sensor transparency of about 0.04mA (the current marker  is fixed as shown in  Fig. 1):

Fig.2: Current through sensor when  resistance value changes

Fig. 3: Simulation settings

Next,  the voltage  across the sensor will be fed  into a  voltage repeater the scheme of  which  is shown below.

V1

U3

LM741

+

3

-

2

V+

7

V-

4

OUT

6

OS1

1

BONE

2

5

-

VCC

V

VCC

V1

U3

LM741

+

3

-

2

V+

7

V-

4

OUT

6

OS1

1

BONE

2

5

-

VCC

V

VCC

Fig.4 Voltage repeater

The voltage measured on  the sensor when scanning the  value of its  resistance is shown in the  graph below:

Fig.5   Measured sensor voltage

As can be  seen,  the  characteristic is linear,  which was expected, and the  voltage values are drained in the range of  6.53-10.76 [V].

## Convert your domain

We  will convert the range  using the  differential amplifier, which has 2  input positions: one is the  voltage on  the sensor, shown in Fig. 5,  and the other is   a  DC voltage of 6.5V which is chosen  according to the  minimum value of  thevoltage measured on  the sensor (6.53V in my  case)  in order to  obtain a range of this value  be as close as possible   to 0V.

The circuit that converts the  domain is taken from  the Electronic  Devices courses  (the course "OPERATIONAL AMPLIFIERS"  to  be exact). To obtain  the  DC voltage  of 6.5V we used a  voltage diffir diffir, being also a   very simple method, and we dimensioned the  resistors according to the formula (3) . The 2 circuits (diff. and voltage divider) are shown below:

0

Rd1

V2	Rd2

VCC0

1k

1k

Fig. 6  Operational amplifier and voltage divider

To size the  resistances of the differential amplifier  we used formula (4) which

is taken from the same DE course. From n calculations, we obtained the  ratio RR 8 =  RR  6  = 2.6

RR5	RR16 required to convert the range  measured on the sensor to the range (0   , VVCCCC − 2) V, using formula (4). We considered a  very high value of resistors in AO diff.   to limit the incoming current as  much as possible.

V1 – the voltage from the voltage repeater outputvoltage from the

V2 −  6.5V  voltage obtained from  the voltage divider

## Setting threshold voltages

In order  to establish the threshold voltages, we need to  know the  resistance of the   sensor to the starting and  shutdown temperatures   of the boiler. In the    specifications the measurable temperature plat is 70 oo C (−30oo ... +40oo). Applying the simple rule of 3 (5) get a percentage of 14.28% to calculate the resistance from 10oC to 10oC.We need to measure this way, because  the boiler starts at −20oC and stops  at 20oC.

To calculate    resistance   to  certain temperatures, use the formula (6), where p is the  percentage of the formula (5) multiplied by a constant (1<c≤ 7)  to arrive at 100%.

Having found out the  resistance of the sensor to certain temperatures,  it remains to calculate the  voltage at  its  terminals with  Ohm's Law, knowing the current(given by the current source  and calculated) above).  To simplify the understanding of setting voltage  thresholds I will make a table.

Tab. 1. Voltage value  across the sensor as a function of temperature

| p[%] | 0 | 14.28 | 28.56 | 42.84 | 57.12 | 71.4 | 85.68 | 100 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TTooCC | -30 | -20 | -10 | 0 | 10 | 20 | 30 | 40 |
| R[kΩ] | 13 | 14.428 | 15.856 | 17.284 | 18.712 | 20.14 | 21.568 | 22.996 |
| Vcalc[V] | 0.078 | 1.742 | 3.38 | 4.989 | 6.552 | 8.086 | 9.594 | 11.076 |
| Vreal[V] | 0.088 | 1.76 | 3.404 | 5.004 | 6.579 | 8.126 | 9.635 | 11.128 |
| LED | ON | ON | ON | ON | ON | ON | OFF | OFF |

Vcalc – represents the voltage  calculated at  the output  of the differential AO  using formula  (4) and considering VV2 in  the formula as  the  voltage across  the sensor at  the output from  the repeater.

Vreal − represents the  actual (measured)  voltage across the sensor (See Fig. 7).

Fig. 7. Voltage on sensor after domain conversion

With this data presented in Table 1., it is observed that threshold voltages  have the values 3,404V (at −10 oo DC) and 8,126V (at +20ooDC) respectively.

To obtain these 2 thresholds we will use tocall an inverting comparator with hysteresis taken from the Electronic Devices course   ("Operaționale_2020 Amplifiers" – series A).

R

1

V

i

+

Is

-

Is

V

A

R

P

V

Ref

Vo

0

+

Is

You

V

threshold

-

up

-

Is

V

threshold

-

low

1

P

Ref

P

RR

V

R

+

⋅

R

1

V

i

+

Is

-

Is

V

A

R

P

V

Ref

Vo

0

+

Is

You

V

threshold

-

up

-

Is

V

threshold

-

low

1

P

Ref

P

RR

V

R

+

⋅

Fig.6 Noninverting comparator  with hysteresis

VCC	0

1k	866

Fig. 7 Hysteresis comparator  and voltage divider for Vref

For calculating the  resistance values we used the  following formulas (7) and  (8) from the Electronic  Devices course   ("Operaționale_2020 Amplifiers" – series A), and for  the voltage divider  formula (9).

From  the project specifications we know  that we must   maintain an enclosure temperature within the  range (−10oC... +20oC).  To achieve  this we arbitrarily chose  a  reference voltage of  6V (Vref) and one of the  resistors of 1k Ohm (Rp), then, knowing the  thresholds (3.404V and 8.126V, respectively), we also determined the   value of  resistance R1 in Fig. 6 as 0.436kOhm. Since this value  is not  in one of the  standardized series, we chose a  value as   close as possible that can be  seen in Fig.7 (0.442kOhm). Again,  we want as  little   current as possible  at the entrance to the   Comparator and multiplied  the  order of the  calculated  resistances until we reached the value of 1MOhm for Rp, respectively 442kOhm for R1.

In Fig. 8 we can see the calculations made and the values obtained.

Fig. 8 Determination of thresholds

It can also  be seen that the lower threshold has  a  voltage value of 4.178V,  which does  not bother, because  this would mean that the plant will  start a little before   that  the temperature  reaches −10ooCC.

## LED modeling

In the project specifications, the  color of the LED to be modeled is MOV. Thus, we chose the LED with  catalog sheet [x] produced by the company Luckylight.

For modeling I followed the  laboratory  tutor from  CAD Techniques (laboratory 7).

Next I will add  more screenshots with  the steps taken.

Fig. 9 Feature taken from  catalog sheet

Fig. 10 Values taken from  catalog sheet

Fig. 11 Parameters resulting from the  simulation tool

Fig. 12 Circuit used for LED testing

Fig. 13 Adding the newly created  library to the simulation profile

Fig. 14 Introducing the  newly created  library into the diode model

Fig. 15 LED  simulation  settings

## Central Relay

The   circuit simulating  the relay driving  the  boiler is composed of a bipolar transistor Q2N2222 that has a  resistance   representing the resistance of the  boiler and   the voltage source connected in collector. The base terminal  of  the transistor is connected to the  output of the  hysteresis comparator, being driven by it, and the emitter is   connected  to grounding.

In addition, at  the exit of the comparator there is  a MOV LED  , sized as follows

catalog sheet, which has a   current-limiting resistance(Rlim = 300Ohm) and indicates the  operation of the boiler.

Q4

Q2N2222

VCC

0

Vo

diodes1

LED

Power station

k

1

Rlim

301

0

Q4

Q2N2222

VCC

0

Vo

diodes1

LED

Power station

k

1

Rlim

301

0

Fig.16 Relay circuit  and  thermal power plant

# Circuit simulation

## Voltage on  temperature sensor

We will perform a  DC Sweep analysis to see how the variation in  sensor resistance influences the measured voltage  on it. In Fig. 17 you    can see  the simulation settings, where you can see the  resistance sweep simulating the  sensor starting from 13k Ohms  up to 23k  Ohms  with an increment of 0.3k  Ohms.

Fig. 17  Sensor simulation settings

We now expect to  see a  linear variation characteristic (Fig. 18) of voltage across  the sensor, which will be converted to the  range specified in the  project requirement.

Fig. 18  Sensor voltage variation

We increased the  simulation range to 24k Ohms to be able to  read the  voltage on the sensor as  clearly as possible. Thus,  the voltage across the sensor is now in the range (6.53V- 10.766V) very similar to that required in the project requirements.

## Voltage dome conversion

Conversion of the voltage range  on  the sensor to the  range specified in  the  requirement can be  achieved with a  differential amplifier. In Fig. 6 you can see this circuit  along with  the sizing of its  components.

To simulate the operation of this circuit, we will use the  same data as  in Fig. 17.

Fig. 19 Voltage range  after conversion

From Fig. 19 It can be seen that the  new domain is between  (0.19V – 11.1V), being similar  to the one in the requirement. It does not reach  the 0V tense because V 1 and  V2 in the formula (4) They can  never be equal.

## Simulation of thresholds

In order to  function properly, the circuit needs 2  thresholds (asymmetrical) The theoretical part of calcuation and sizing of  thresholds was made above  (Fig. 8).

Due to the  fact that the simulator does not allow us  to visualize the   hysteresis in the  same simulation, we will do 2 simulations:  one in which we will scan the  resistance of the sensor upwards and one in which we're going to swing the other way around.

The simulation settings  are identical as in Fig. 17.

Fig. 20 Left threshold  of hysteresis

Fig. 21 Right threshold  of hysteresis

As can be seen in Fig. 20, the left threshold is  reached when the  voltage on  the sensor is about 4V and the  resistance value  is  16.4k Ohm,  which is not bad, even  if it is  not  the calculated  threshold, because this means that the boiler will start at  a temperature of −10oC.

In Fig. 21 it is seen that the 2nd threshold is reached at the voltage determined in Fig. 8., the boiler turning off when the  temperature reaches +20oC and the resistance of approx. 20k Ohms.

## LED simulation

To check the  functioning of the  LED we will measure the current   passing through it, at  the variation of the  resistance of  the temperature sensor. In the  figure below  it is seen that the  LED is on (I>0A) when the  boiler is  turned on (  the temperature is not  in the  desired range) and   is   off (I~0A) when the boiler is not operating (temperature belongs to desired range).

Fig. 22 LED current

## Monte-Carlo Analysis

Monte-Carlo  analysis is the  best way to analyze a circuit statistically  and show you how it behaves when  certain components vary. It also gives us  an almost real picture  of  circuit operation in series production, where each component has a certain tolerance.

In the circuit  required in  the  specifications, I consider  that  a  very large influence has the resistance that sets the polarization current of  the  sensor, more precisely R2 in the electrical diagram  of the circuit.

If we perform a  Monte-Carlo  simulation and set the  parameter TOLERANCE = 10% for this resistance, we will see that the do menu of the  voltage measured on the sensor changes  (Fig. 24).

Fig. 23 Monte-Carlo  simulation  settings

Fig. 24 Change in  measured sensor voltage

The DC  Sweep  simulation settings are the same as  in Fig. 17.

Also,  the  voltage range  that has been converted  to vary in the range

(0, VCC − 2) increased, reaching  (Vcc − 0. x )[V], i.e. more than 1V dthe previous maximum.  This is shown  in  the  figure below  .

Fig.  25 Maximum voltage measured at   AO-diff output  doing domain conversion

## Worst-Case Analysis

This analysis shows which   circuit components  are critical to circuit operation. This analysis is also  performed for sensor resistance.

Fig. 26  Worst-Case  analysis settings

Fig. 27 Characteristic following  Worst-Case  analysis

In Fig.   27 it is observed how much the  sensor tolerance  (10%  in  this case)  influences the  maximum output voltage in the  differential amplifier   in Fig. 6 that performs the  conversion of the voltage range.

# Bibliography

- (2023) The IEEE website, https://3dstar.ro/calculator-divider-voltage

- (2023) The IEEE website, http://www.bel.utcluj.ro/dce/didactic/cef/

- (2023) The IEEE website, http://www.bel.utcluj.ro/dce/didactic/de/de.htm

- (2023) The IEEE website, https://site s.google.com/site/bazeleelectronicii/home/rezistoare/2-resistor colour-code

- (2020) The IEEE website, https://www.pspice.com/

- Laboratory tutor  from CAD Techniques

- "Q2N2222 data sheet", ETC

- "LM741 data sheet", Texas Instruments, Texas, USA

- "BC177 data sheet", STMicroelectronics

- "1N5225 data sheet", General Semiconductor